Global Conservation Standard

Tool Number 2:

Methodology for Forest Conservation Projects

v

31 December 2009

## TropBio Carbon Exchange Sdn Bhd

## March 2004

biocarbon consult

Telephone: +49 (781) 970 5749

Email:	info@biocarbon.net

URL:	http://www.biocarbon.net

© biocarbon Consult, 2009. All Rights Reserved.

# Purpose

The Global Conservation Standard (GCS) is a carbon accounting system, which is based on carbon stocks, not variations. Buyers of GCS credits take over stewardship for the vegetation, but will not be able to offset any emissions with the certificates. GCS is however designed in a way to be compatible with offset-creating project types.

This methodology gives guidance on how to account for carbon conservation in forest ecosystems. It covers the issues of project eligibility and exclusion criteria, conditions for application, definition of activities, and project boundaries.

# Eligibility Criteria

For being eligible under GCS, projects need to fulfill the following requirements:

- The proponent organization proposing the Forest Conservation Project must clearly define the ownership of land management rights of the conservation area for long-term conservation.

- The proponent organization must demonstrate legality of land use for conservation in conjunction with any valid land claims by local communities or native rights that overlap with the proposed conservation area.

- The net conservation area and associated value of carbon stocks as determined through the PIN assessment must be economically viable to the project owners, CCF & In Country Foundation as well as deemed environmentally and socially beneficial.

- A Conservation Contract is signed with CCF that excludes land-use change for a period of at least 30-years

- An carbon stock inventory is carried out annually, as per the Conservation Contract and this methodology

A failure to comply with these criteria will lead to corrective action requests by GCS.

# Boundaries and Area Definitions

Carbon monitoring, reporting and verification of conservation activities are strictly area-based. GCS distinguishes between the following area definitions:

Table 1: Area Definitions

| Conservation Area (CA) | The Conservation Area is the forest area as defined in the Conservation Agreement that is to be protected as a natural forest area to serve as carbon storage |
| --- | --- |
| Reference Area (RA) | An area that encompasses a strip of between 10 and 50 km around the CA (to the discretion of the Technical Panel). This Reference Area can be used for observing direct local influence on land use (for observing activity leakage and baseline) |
| Political Influence Area (PIA) | Depending on project size and governance structure and data availability, the province or the country, where the CA is located. |

Supporting activities will take place on a wider area than the protected area. GCS is explicitly based on CASE (Conservation-Activated Sustainable Enterprise), a landscape approach that takes conservation as a centerpiece for local development. Supporting activities shall be initiated by the Options and Choices Foundation that benefits from the conservation project. Most likely several such activities will be combined to appropriately support the Conservation Activity. Table 2 gives an indicative list of supporting activities that may be taken by the project implementers. The Options and Choices Foundation will propose appropriate indicators by which the effectiveness of supporting activities can be assessed.

Carbon benefits arising from supporting activities will not be reported under GCS, but they may be reported under a third-party verification scheme. All forestry operations undertaken as supporting activities will eventually have to come under forestry certification from an internationally recognized standard. None of the supporting activities may lead to land conversion from forested to non-forested land, unless previously agreed in the Conservation Agreement.

Table 2: Indicative list of supporting activities and appropriate indicators

| Supporting activities | Indicative success criteria |
| --- | --- |
| Forest management in adjacent areas | Sustainable timber supply |
| Afforestation / reforestation for timber production | Sustainable timber supply |
| Biofuel production | Sustainable biofuel supply |
| Agroforestry / silvopastoral systems | Area put under management |
| Intensification of existing agriculture or animal husbandry | Increase of volume or value of products on a constant production area |
| Secondary processing agricultural / forestry products | Introduction and/or enhancement of secondary production |
| Research and development | Initialization of research and educational programs |
| Ecotourism development | Number of visitors Income generated by tourists |
| Training and capacity building | Number of persons trained under different programs |
| Public health improvement | Adequate provision of clean water per capita, increase of proper sanitation Number of persons benefiting from improved healthcare |

# PIN Carbon Stock Estimation

A first-order estimate of existing carbon stocks will be undertaken along Tier-1 criteria of the Intergovernmental Panel on Climate Change (Eggleston, Buendia et al. 2006), only considering aboveground carbon stocks. Proponents are required to truthfully complete the PIN and associated Spreadsheet. The first-year GCS credits sales will be based on this estimate and an independent uncertainty assessment by the GCS Technical Panel.

The following stepwise approach is taken:

- Document the area with remote-sensing data

- Stratify and map the area along ecosystem and state of degradation using GIS

- Measure the size of each area

- Input these data into the PIN and the carbon estimation spreadsheet provided

# Carbon Monitoring Methodology

Subsequently, a monitoring system will be put in place that combines remote sensing with on-the-ground carbon measurement.

Monitoring consists of three elements:

- Carbon stock measurement within the project area

- Carbon stock variation assessment in the reference area

- Project activity reporting

## Carbon Stock Measurement

Carbon monitoring consists of annual monitoring of the forest area based on forest type and risk of encroachment. Forest types and quality strata that are identified and mapped in the baseline evaluation will be monitored for carbon based on satellite imagery and a series of permanent sample plots (PSP) (MacDicken 1997). The number of PSPs in each forest type will depend on the area and the variability in the forest quality in each type. The greater the area and variability, the larger the number of PSPs required for monitoring of carbon within the forest type.

Forest Types are defined under IPCC as listed in “Tool Number 3: Carbon Calculation Spreadsheet” to be used in defining the Conservation Area for the Project Idea Note (PIN).

Forest Quality Strata is defined as the current status of the forest volume relative to the potential forest volume of an undisturbed virgin forest on the same site quality.

GCS uses 4 distinct levels of site quality:

- Natural Forests:

Forests that contain volumes of 75% or more of the potential volume that can be supported by the site

- Secondary Forest:

Forests that contain volumes of 50-74% of the potential volume that can be supported by the site

- Degraded Forest:

Forests that contain volumes of 25-49% of the potential volume that can be supported by the site

- Severely Degraded Forest:

Forests that contain volumes of 10-24% of the potential volume that can be supported by the site

### Risk Association by Strata

Evaluation of risk can be ascertained by review of satellite imagery in the project Conservation Area and Reference Area as well as through social evaluation of local communities that may experience potential significant impacts on the project area. In general areas that are degraded have been demonstrated to consist of a High Risk as they tend to more accessible and closer to local communities. Areas that have well developed natural forests tend to be low risk due to larger distance from local communities and poor access. Often this is the case where greater disturbance occurs in lowlands and forests are least disturbed in steeper areas. Currently, the Standard is using forest quality by strata as an element of its risk assessment.

High Risk: Based on observed significant impacts and potential for encroachment by communities on the project Conservation Area; Usually consists of degraded areas that have been repeatedly logged in low land areas that have easier accessibility. Forest Strata Association: Degraded & Severely Degraded Forest

Medium Risk: Based on observed partial or inconsistent demand for forest resources from local communities that could be obtained from the conservation area. May contain areas that have had some history of logging or encroachment but still have significant standing timber volumes. Forest Strata Association: Secondary Forest

Low Risk Based on insignificant or no observed demand for use of forest resources from the Conservation Area. Usually contain areas of well develop natural forests often on steep terrain with poor accessibility Forest Strata Association: Natural Forest

### GIS Imagery Application

Evaluate the project Conservation Area by satellite imagery as deemed necessary to evaluate risk within the Conservation Area or Reference Area. This includes obtaining imagery at the scale of 1:25,000 for project areas as necessary.

Satellite imagery should be used to identify newly encroached areas. Measurements to evaluate existing volume of forest following encroachment are to be conducted by strip plots 10x100 m with a minimum of 20 plots or as deemed required to obtain a credible average volume.

Satellite Imagery is to be conducted annually along with the monitoring process.

### Sampling Intensity

GCS goal is to obtain reliable inventory data through a series of permanent sample plots in each forest type and quality strata to monitor carbon stocks within the forest conservation project area (MacDicken 1997), (Muller-Dombios and Ellenberg 1974). As degraded forests tend to be more variable in tree density and have associated higher risk of sampling intensity can be in proportion to the area in each quality strata.

However there also must be a limit on sampling to meet limitations in budgets and manpower to conduct repeated sampling within conservation areas. The maximum number of sample plots that can be properly measured and maintained on an annual basis within the forest conservation project is anticipated to be 400 plots.

Natural Forest (Low Risk): 				1 sample plot for every 300ha of strata

Secondary Forest (Medium Risk):			1 sample plot for every 200ha of strata

Degraded and Severely Degraded (High Risk): 	1 sample plot for every 100ha of strata

Projects are required to sample all strata found according to GIS and field surveys.

### Permanent Sample Plot (PSPs) Establishment

Permanent Sample Plots (PSPs) are established using circular plots of 20 m radius whereby all trees over 10 cm diameter (DBH) are located, numbered, marked and measured to 0.1 cm accuracy using a standard diameter tape.

PSPs must be located with some element of randomness within each forest strata that is also functional in respect to accessibility for re-measurement. In strata where there is high risk, a higher concentration of PSPs is expected to be located where there are historical or current risks to the project area (i.e. road access to project site). PSPs are to be numbered and mapped along with GPS coordinates.

PSP center point must be located with GPS coordinates (WGS Datum) and clearly marked.  All trees starting from 0o to 359o shall be measured for diameter at the standard 1.3m above ground level. All living trees 10cm DBH and above within 20m of center point shall be recorded. Every second tree that is on the boarder of the 20m radius shall be measured. First boarder tree shall be measured 2nd shall be skipped 3rd measured etc.

Marking of location for DBH measurements must be clearly painted on each tree following measurement to identify re-measurement locations.

Measurement records include: bearing, distance from center point (meters), species and DBH (cm). All PSP data needs to be recorded using a foresters diameter tape and entering the findings in the form provided in Appendix I: PSP Template for Carbon Monitoring. All data must be transferred into electronic format using Excel data sheets.

Measurement of all established PSPs are to be conducted annually.

### Permanent Sample Plot Volume and Carbon Calculation

Based on the PSP data, determine the carbon inventory for the project Conservation Area.  Formula for woody biomass calculation for each permanent sample plot (PSP).

Record all trees, corresponding and DBH to determine volume of carbon in the PSP. The PSP template is located in Appendix I below.

## Leakage and Carbon Flow Monitoring

A second layer of carbon monitoring concerns the baseline development in the assumed absence of the project. This is to qualify how additional the Conservation Activity is to continued inaction, and whether it produces carbon leakage in surrounding areas. The result of this monitoring stage does not influence the overall number of GCS credits, but it will eventually influence their sales value.

The following steps shall be taken:

- Determine the country deforestation rate in the PIA, as officially communicated (FAO). If available and appropriate, the provincial deforestation rate can be used, if the PIA is defined to represent the province level.

- Using GIS, classify the RA along the categories “natural forest under different stages of management or degradation”, “planted forests” and “non-forested land”.

- Beginning from the second year, compare

- The development of the deforestation rate in the PIA, and

- The development of the deforestation rate within the RA.

- Longer time series are usually more reliable; five-year running averages will be preferred, as soon as data become available

- In case the deforestation trend (not the absolute deforestation rate) is higher in the RA than it is in the PIA, the project is likely to produce negative leakage. Under this scenario, project proponents are required to refer to the Technical Panel.

- In case, no negative leakage is determined, applying the RA deforestation rate on the CA will allow reporting the amount of carbon not released during the monitoring period

- Annual baseline and leakage determination is a vital fact for credit valuation, and it will be starting in the second year. This report may include explanations on the forest situation within the RA, like the occurrence of dry periods, thunderstorms, pests, wildfires or other factors outside the influence sphere of the project participants.

As different from REDD activities, the fact that there is deforestation or degradation pressure on the area does not constitute a requirement for the generation of GCS credits. It is however an indication, whether the activity has been effective. Even though there might not be an immediate relief of pressure on the area, the ongoing protection may become effective once in the future deforestation and degradation move near the CA.

## Activity Monitoring

Carbon monitoring will be undertaken annually and will be summarized in an annual report The report is considered part of the monitoring process, forming an integral link for SFM and certification as a whole.

# References

Eggleston, H. S., L. Buendia, et al., Eds. (2006). Agriculture, Forestry and Other Land Use, 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Prepared by the IPCC National Greenhouse Gas Inventories Programme. Hayama, Kanagawa / JP.

MacDicken, K. G. (1997). A Guide to Monitoring Carbon Strage in Forestry and Agroforestry Projects. Arlington, VA, USA, Winrock International: 87.

Mueller-Dombois, D.and H Ellenburg. 1974. Aims and Methods of Vegetation Ecology. Wiley & Sons. 557 pp.

Appendix I: PSP Template for Carbon Monitoring

To be included into the revised draft after the initial review